“STREAMcom”

ODD

STREAMcom Version 3.3.0

05.03.2024

Author(s)

Jana Gerhard
Tido Strauss
Josef Koch

Table of content

1.	Purpose	3

3.	Process overview and scheduling	6

4.	Design concepts	8

4.1	Basic principles	8

4.1.1	Temperature dependency	8

4.1.2	Super-individual approach	9

4.2	Emergence	9

4.3	Adaptation	10

4.4	Sensing	10

4.5	Interaction	10

4.6	Stochasticity	10

4.7	Observation	11

5.	Initialization	11

5.1	Initialization of the environment	12

5.2	Initialization of the community	13

6.	Input	15

7.	Submodels	17

7.1	Individual life history processes based on DEB theory	17

7.2	Process based TK-TD models for lethal and sublethal effects	24

7.3	Calculation of initial resources	29

7.4	Update resource densities	30

7.5	Calculation of periphyton growth	30

7.6	Calculation of particulate organic matter	32

7.7	Movement	34

7.8	Feeding herbivores	34

7.9	Predation	37

7.11	Reproduction	38

7.12	Survival	40

8.	References	42

This model description follows the ODD (Overview, Design concepts, and Details) protocol (Grimm et al., 2020).

- Purpose

The STREAMcom has been developed at gaiac during the last years within a CEFIC project (2015-2018) to simulate macrozoobenthos communities. The life history cycle of each species within the individual-based population models are implemented based on dynamic energy budget (DEB) theory. All individuals interact within the total benthic community of this IBM approach.

The individual-based STREAMcom model (IBM) integrates information on habitat scenarios, functional traits and life history parameters (DEB) of individual species, and chemical effects (TKTD) to simulate community dynamics of benthic invertebrates temporally and spatially.

Although STREAMCom was originally designed for running water communities, it can also be applied to standing water bodies with only minor adaptations in the environment.

- Entities, state variables and scales

The model includes the following six entities: individual, spatial unit (cell), environment, population, community, resources.

The individual object includes all state variables required to compute the individual life cycle. The metabolic processes of the individuals are based on the DEB-Theory. A detailed overview of the DEB related parameters and the DEB model can be found in Section 7.

The spatial structure of the scenario is realized through a structural and functional parametrisation of single grid cells. The size and trait of the cells is parametrized by the scenario map, which is read from an external text file. Each cell is embedded in the landscape grid and characterized by its X/Y coordinates. The simulated scenario uses closed boundaries that represent the spatially explicit landscape derived from observations and mapping. A cell is further characterized by its environmental factors (i.e. microhabitat, velocity and resource content), which are determined by the study site and potential scenarios.

The entity of ‘environment’ refers to the overall environment and does not vary across space. It drives the behaviour and dynamics of all agents and grid cells.

The entity ‘population’ contains the species abundance and spatial distribution, while ‘community’ contains an overview of the entire community from all populations, including also dead bodies.

The entity ‘resources’ is a collective that characterizes the specific parameters and functions that control the behaviour of the different resource classes. An overview of the input parameters is given in Table 1.1 and Table 1.2.

Table 1.1: Input parameters of the entities ‘Individual’ and ‘Environment’

Table 1.2: Input parameters of the entities ‘Cell’ and ‘Resources’

Scale. Time in the model is run in days. Thus, the user can specify the number of days, e.g. 10 or 100 days, and run the simulation. In a simulation, most of the processes will occur a daily time step (e.g. resource calculation), while the DEB processes and the calculation of feeding rate will occur at hourly time steps.

# Process overview and scheduling

An IBM approach is used to simulate the life cycle of each individual macroinvertebrate or fish in the simulated water body. The schedule of the life cycle is shown in figure 3.1. Most of the procedures are based on daily time steps, the calculation of the feeding rates and the processes of the individual life cycle (growth, maturation, reproduction, survival), only, are run in hourly time steps. The effects of toxic substances can be calculated on a daily or hourly basis.

The simulation sites are mappings of existing research areas. A detailed overview of the initialization of a simulated site is given in section 5. Resource density is calculated daily as a function of temperature, light, velocity and settlement rates (section 7.5).

The individuals perceive the environment, including the environmental temperature, the photoperiod of the simulated day and the exposure concentration. The exposure concentration is the only one of these values that is recorded hourly, the other conditions are recorded daily.

Individual movement depends on a probability of leaving the current patch. It is computed based on the scores derived from the microhabitat preferences and the feed condition of the individual. The individuals can move only one cell per day.

Each simulated individual selects a food type based on the energy density of the food types on the visited patch and a selectivity score derived from the species trait distribution. Each individual may feed on only one food type per time step (day). Diet switches are possible if, due to feeding activity, the energy densities of resource types change relatively to the selectivity scores or if the animal leaves its current patch.

The life history of individuals depends on the actual food intake, the ambient temperature and the length of the day. They grow, reproduce and eventually die, depending on aging or starvation (e.g. in competition situations). Some species can reproduce throughout the whole year if the food availability and temperature is sufficient, others reproduce in seasonal patterns triggered by environmental factors. The entire life cycle of individuals is simulated on the basis of DEB theory. A detailed description of all DEB processes can be found in section 7.1.

Once the individual life cycles are simulated and the number of dead individuals per day is calculated, the population sizes of the different species are updated.

Figure 3.1: Flow chart of the processes within individual life cycle. Flowchart diagram for the STREAMcom population model (black arrows). The ecological module includes an IBM (green) with the physiological DEB module (yellow) coupled to a toxicological TKTD module (grey). The DEB-TKTD module is schematically simplified as a box

- Design concepts

## 4.1	Basic principles

The purpose of the model is to simulate community dynamics of freshwater macroinvertebrates in temporal and spatial terms. Therefore, ecological traits are used to describe the structural and functional diversity in the model and to inform the modelled organisms about habitat selection and feeding interactions. The calculation of the individual life cycle is based on DEB theory. In addition, the model can predict lethal (TKTD) and sublethal (DEB-tox) chemical effects on populations.

## 4.1.1	Temperature dependency

To account for temperature dependency of the modelled processes, a temperature correction factor TC (−) was used, which is consistently applied to all physiological rates:

|  | TC=expTATREF-TAT, &T<TREFsHexpTATREF-TAT, &T≥TREFsLexpTATREF- TAT, &T≤TREF | (4.1.1) |
| --- | --- | --- |

with the upper boundary correction sH (−):

|  | sH=1+expTAHTH-TAHTREF1+expTAHTH-TAHT | (4.1.2) |
| --- | --- | --- |

and the lower boundary correction sL (−):

|  | sL=1+expTALTREF-TALTL1+expTALT-TALTL | (4.1.3) |
| --- | --- | --- |

Within the reference temperature TREF (K), the temperature dependency is described by only two parameters: TREF and the Arrhenius temperature TA (K). At and above TREF, two additional parameters, i.e., the upper boundary temperature TH (K) and the Arrhenius temperature for the upper boundary TAH (K), are used to model a flattening of the temperature curve where physiological tolerance is exceeded. At and below TREF, the lower boundary temperature TL (K) and the Arrhenius temperature for the lower boundary TAL (K), are used to model a delay of the temperature curve where physiological tolerance falls below the optimal temperature range.

## 4.1.2	Super-individual approach

To enable the simulation of large populations, a super-individual approach was implemented within the model. A super-individual is an individual object representing a set of siblings that share a location and life history processes. To reduce run-time and memory, super-individuals are created above a certain population size threshold and split back into individuals when the population size drops below the threshold to avoid super-individuals to bias results at low densities. Accordingly, super-individuals and individuals may be present simultaneously during the course of the simulation. The population size threshold is set at 10,000,000 individuals and the number of individuals represented by a super-individual is set to 70; both values can be changed by the user.

When a new individual is created, the population size determines whether the individual becomes a normal individual or a super-individual. As soon as the population size drops below the threshold, the existing super-individuals split into normal individuals.

## 4.2	Emergence

Individual life history traits as well as population dynamics result from metabolic organisation (the organism with its metabolism), dispersal behaviour and intraspecific competition among individuals for food.

In its current state, the model provides the following community and population endpoints. The size [#], or optionally the density [#/m²], including the age distribution of each simulated population over the simulation time. The emergent rates for emerging insects over the simulated time. The spatial distribution of individuals per cell, divided at the end of the simulation according to their habitat and feeding characteristics. The causes of death (e.g. starvation, aging) for each individual.

These key outcomes emerge from the competition within and between the different populations, the life cycle processes of the individuals, the disturbance by chemicals, and the effects of the environmental conditions.

## 4.3	Adaptation

The movement probability of individuals is influenced by their habitat preferences and their foraging procedure. Thus the probability of movement increase if the individuals dislike the conditions (microhabitat, food type) on the current patch or if they have low energy reserves.

## 4.4	Sensing

Most of the sensed parameters are recorded by the animals once a day, including water temperature, photoperiod and the habitat and food categories of the cell in which they reside. The only exception is the external exposure concentration, which is sensed every hour.

Only the habitat and food categories directly affect the behaviour of the individuals, as they influence the probability of movement. All other parameters affect the life history processes of the DEB and TKTD model and are responsible for the change in life parameters, e.g. reproduction probability, hazard rate, maintenance costs etc.

## 4.5	Interaction

The predation procedure of predatory species (see 7.9) is the only direct interaction between species implemented.

## Stochasticity

To take natural variation into account, several processes are calculated stochastically:

Mortality is assumed to be a probabilistic process. For each individual, a survival probability is calculated, based on modules for ageing and starvation. In addition, we implemented a background mortality and a density dependent mortality for each species, these mortalities are independent of all DEB and TKTD processes. An individual survives if the survival probability exceeds the calculated mortality probabilities from the mentioned mortality causes. Each day, the probability of survival is newly drawn from a uniform distribution ranging between 0 and 1. The second source of stochasticity is related to the movement. In the current state, the direction of movement of the individuals is completely random. Other sources of stochasticity are related to the composition of the start population (see initialisation section below) and to the migration rate (see 7.11).

Every simulation day, the individuals go through the life cycle processes in random order. For the individuals, only the sex and initial length are randomly assigned, all other individual parameters are fixed values.

## 4.7	Observation

Throughout the simulations, we carefully observe the population dynamics, tracking the categorization of individuals into eggs, juveniles, adults, and emergents, as well as their maturity levels and spatial dispersion. In addition, we monitor the abundance of the different resource factions.

  - Downhill-Simplex calibration algorithm

The model includes a Downhill-Simplex Algorithm to calibrate different parameters based on given datasets. The Algorithm was implemented based on the work of (Press et al., 1989). This algorithm is adept at locating the minimum of a function with multiple independent variables. To initiate the minimization process, we provide the algorithm with an initial guess—an N-vector of independent variables. Subsequently, the algorithm navigates through the multidimensional terrain, progressively descending until it reaches a minimum point. This progression involves a series of steps, where the simplex's point with the highest function value moves towards a lower point across the opposite face of the simplex. Termination occurs when the decrease in function value at the final step is fractionally smaller than a predefined tolerance threshold (1E-7). In our approach, we utilize the summed squared deviations as the function values and apply these values at the end of the calibration process as a measure of the goodness for the calibration.

- Initialization

## 5.1	Initialization of the environment

The STREAMcom model is run based on a set of default parameter values and initial conditions for the environmental settings (Table 5.1).

The parameter values were derived from literature (Table 5.1), with the exception of the settlement rates of C-POM and F-POM and the fraction of S-POM available to filter feeders. The settlement rates quantify the proportions of C-POM and F-POM that remain in the stream sections; the other fraction is assumed to drift downstream and is not further considered in the model. Moreover, we assumed that only a part of the S-POM in the water column is available to benthic filter feeders, the unfiltered material is considered to turn into F-POM the following simulation step. Settlement rates as well as the S-POM fraction are assumed to be constant and were adjusted in a way that allows relatively stable population dynamics in the simulations.

Table 5.1: parameter values and initial conditions for the environmental settings

We use raster map representations of the different study sites as spatial explicit habitat information input for the model. Therefore, in a first step, the GIS polygon map layers for the different habitat characteristics mineral substrates, velocity, water depth relative to mid water level, organic substrates and canopy cover are merged. In a second step, the merged GIS polygon maps are transferred into grid matrices with the habitat composition of each raster cell expressed individually as a numeric output. For an example see Figure 5.1.

The initial amount of resources is calculated based on the determined environmental settings. In section 7.4 this calculation is shown in detail.

Figure 5.1: Raster map for a 25 m stretch of the upland stream Vichtbach. Colour coding is chosen in a way that allows the illustration of the habitat diversity which arises from the spatial combinations of the different habitat characteristics and their categories.

## 5.2	Initialization of the community

Simulations are initiated by setting the initial amount of start individuals, either in g/m² or in individuals per m². New individuals are created as long as the summed individual biomass does not exceed the initial species biomass (eq. 5.1) or the selected population size. The start individuals are distributed based on the species preferences for the biocoenotic zone of the selected habitat sections. Species preference is tested for each individual on only 10% of the cells. If no suitable cell is found the individual is not created. In this way, the initial biomass is automatically reduced if the site does not provide optimal conditions for the species. The age distribution of the population is based on the size of the individual created. The size of each individual is drawn from a normal distribution and is used to determine the age of the animal.
During the initialization process the state parameters of the individuals are calculated based on the DEB rules. During this process we assume no energy limitation for the individuals and the initialisation process ends with achieving the individuals start size.

species biomass=initial biomass gm2*AreaAquaticCells m2Number of Species				(5.1)

- Input

The STREAMcom model requires six data files as input. These files contain the species and habitat parameters, the toxicity information, the water temperature, etc. (see Table 6.1).

Table 6.1: Overview input files

| Input files | Content |
| --- | --- |
| spec.xml | - DEB parameter of all species - Habitat preferences - Food preferences - Life-cycle parameters |
| tox_information.txt | - General substance information - Concentration unit - GUTS-SD parameters - Sublethal effect parameters, mode of action |
| settings.txt | - Simulation settings (resources, initial values, adjustable parameters, simulation time, output settings) |
| exposure.txt | - Exposure profile (daily or hourly) |
| temp.txt | - Temperature profile (daily or hourly) |
| site_information.txt | - General properties of the site (waterbody) - Stream type category (LAWA) - grid map of habitat properties |
| Biomass.txt | - Initial biomass or individuals of macroinvertebrate and fish - calibrated parameters: - background mortality - migration rate - half saturation constant |

The list of available focal macroinvertebrate species (Table 6.2) covers a wide range of traits: the zonal distribution ranges from epirhithral to ditches, ponds and lakes, life cycles are multi- to univoltine, all microhabitat preferences are covered except for argylal, all major feeding types, i.e. filter feeders, collectors/gatherers, shredders, grazers, and, to lower extent, also predators are represented by the focal macroinvertebrate species to be considered in the IBM.

Table 6.2: Overview of species in the database

| Species | DEB model | Survival model | Reproduction |
| --- | --- | --- | --- |
| Asellus aquaticus | abj | lpg | (Ovo)viviparous |
| Baetis rhodani | hep | lgp | Semelparous |
| Chironomus riparius | hep | lgp | Semelparous |
| Cloeon dipterum | hep | lgp | Semelparous |
| Dikerogammarus villosus | abj | lgp | Iteroparous |
| Ecdyonurus dispar | hep | sgp | Semelparous |
| Epeorus assimilis | hep | sgp | Semelparous |
| Gammarus fossarum | abj | lgp | (Ovo)Viviparous |
| Gammarus pulex | abj | lgp | (Ovo)Viviparous |
| Gammarus roeselii | abj | lgp | (Ovo)Viviparous |
| Gasterosteus aculeatus | std | lgp | Iteroparous |
| Habrophlebia lauta | hep | lgp | Semelparous |
| Lymnaea stagnalis | abj | lgp | Iteroparous |
| Perca fluviatilis | abj | lgp | Iteroparous |
| Planorbis planorbis | abj | sgp | Iteroparous |
| Potamopyrgus antipodarum | abj | sgp | (Ovo)Viviparous |
| Rhithrogena semicolorata | hep | lgp | Semelparous |
| Salmo trutta | abj | lgp | Iteroparous |
| Sphaerium corneum | std | sgp | Iteroparous |
| Stagnicola palustris | abj | sgp | Iteroparous |

- Submodels

## 7.1	Individual life history processes based on DEB theory

Life histories of individual organisms are modelled based on the Dynamic Energy Budget (DEB) theory (Kooijman, 2010). DEB models describe individual life-history processes based on energy fluxes: Organisms assimilate resources from their environment and subsequently allocate energy through a reserve compartment to maintenance, somatic growth and the reproduction system. In the reproduction system, maturity is built up over time (also increasing the running costs for maturity maintenance) until the life-history event of puberty is reached. After this, maturation ends and reproduction begins (Figure 7.1).

Variables and parameters are listed in Table 7.1 and Table 7.2 respectively whereas model equations are given in Table 7.3.

Depending on the modelled species, we either use the standard DEB model (std) or one of the model extensions abj (metabolic acceleration from birth until the life-history event of metamorphosis, denoted by the letter j) or hep (like abj but offspring is released only once at the life-history event emergence instead of regular brood intervals).

The std-model applies to most animal species without larval phases since it assumes isomorphy throughout the full life cycle.

The abj–model applies to most species with a larval phase and is a one-parameter extension of std.model. The model is like the std-model, but it includes a type Μ acceleration between birth EHb and metamorphosis EHj. Metamorphosis is before puberty, which might or might not correspond with changes in morphology. During metabolic acceleration, {pAm} = {pAmb}L/Lb and v =vb L/Lb, where {pAmb} and vb refer to the values at birth. At metamorphosis, acceleration ceases: {pAm} = {pAmb}sM  and v =vb sM, with acceleration factor sM= Lj/Lb.

The hep-model is a model for hemi-metabolic insects, such as ephemeropterans, odonata and possibly other insect groups. It includes eggs, larvae and (sub)imagos as morphological life stages and embryos, juveniles, adults and imagos as functional stages. The embryo development follows the std-model but between birth EHb and puberty EHp the model assumes type M acceleration. The transformation to subimago occurs when the reproduction buffer exceeds a thresholdERj. So isomorphic growth occurs in adult nymph, but not in the (sub)imagos.

Figure 7.1: Schematic representation of the standard DEB model and exemplary physiological modes of action (see below); modified from Álvarez et al. (2006).

Table 7.1: Variables and intermediate outputs for modelling life histories.

| Symbol | Parameter description | Unit |
| --- | --- | --- |
| Ue | scaled reserve level | [-] |
| Sc | scaled mobilisation flux of reserves | [J / d*cm²] |
| e | scaled reserve density | [-] |
| eb | scaled reserve density at birth | [-] |
| L | structural length | [cm] |
| pX | feeding rate | [-] |
| f | scaled functional response | [-] |
| R | reproduction rate | [/d] |
| UH | scaled maturity | [d*cm²] |
| UR | scaled reproduction buffer | [d*cm²] |
| q | aging acceleration | [/d²] |
| h | hazard rate | [-] |
| s | survival probability | [-] |
| hs | hazard rate for starvation | [-] |

Table 7.2: Parameters for modelling life histories.

| Symbol | Parameter description | Unit |
| --- | --- | --- |
| g | energy investment ratio | [-] |
| km | somatic maintenance rate coefficient | [/d] |
| v | energy conductance | [cm/d] |
| kappa | allocation fraction to soma | [-] |
| kr | reproduction efficiency | [-] |
| kj | maturity maintenance rate coefficient | [/d] |
| E_Hb | maturity at birth | [J] |
| E_Hp | maturity at puperty | [J] |
| E_Hj | maturity at metamorphosis | [J] |
| E_m | Reserve capacity | [J] |
| pAm | maximum specific assimilation rate | [J/(d * cm²)] |
| pXm | maximum specific feeding rate | [J/(d * cm²)] |
| kappa_x | digestion efficiency food to reserve | [-] |
| UBH | scaled maturity at birth | [d * cm²] |
| UJH | scaled maturity at metamorphosis | [d * cm²] |
| UPH | scaled maturity at puperty | [d * cm²] |
| E_0 | initial reserve | [J] |
| U0E | cost of an individual egg/embryo | [d * cm²] |
| dm | shape coefficient | [-] |
| zoom | zoom factor | [-] |
| ha | Weibull aging acceleration | [-] |
| sG | Gompertz stress coefficient | [-] |
| Fm | maximum specific searching rate | [l/(d * cm2)] |
| sM | acceleration factor | [-] |
| Lb | structural length at birth | [cm] |
| Lp | structural length at puberty | [cm] |
| Li | structural length ultimate | [cm] |
| E_Rj | Reproduction buffer at emergence | [J/cm³] |
| TA | Arrhenius temperature | [K] |
| TAL | Arrhenius temperature lower boundary | [K] |
| TL | Temperature for lower boundary | [K] |
| w | density of dry weight | [g/cm³] |
| xi_WE | whole-body energy density | [J] |
| Ls | Length at start of acceleration | [cm] |
| Le | Length at end of acceleration | [cm] |

In this model, the acquisition of resources from the environment depends on the environmental food density x. We use a functional response type II (Holling, 1959) for modelling food ingestion. Many modifications to the type II function have been suggested (Jeschke et al., 2002); a general interpretation for the ingestion rate pX can be based on a maximum ingestion rate, which in turn is determined by the squared length L of an individual times the surface-area-specific maximum ingesting rate pXm (eq. 1). The ingested food is taken up into the body with some efficiency kappa_x, which is usually assumed to be constant for a particular type of resource (van der Meer, 2006) and is independent of body size (Hendriks, 1999), to derive the assimilation rate pA (eq. 2). The DEB scheme makes use of a scaled functional response f that can exhibit values between 0 and 1. f can be computed by dividing the assimilation rate by the (surface-area-specific) maximum assimilation rate times the structural surface area (eq. 3).

It should be noted that for all further energy allocations within the organism, we use the scaled DEB model. By dividing all state variables and fluxes of energy (which can also be expressed in units of mass) by the (surface-area-specific) maximum assimilation rate, the units of energy or mass are removed from the model.

Assimilated energy is allocated to a reserve compartment. The scaled amount of reserve Ue depends on the mobilization flux Sc and with the onset of feeding at birth, also on the assimilation flux. For embryos, pX and pAm are 0 (eq. 4). Under growth conditions, the mobilization flux Sc depends on the length L, the energy investment ratio g, the scaled reserve density e, the energy conductance v and the somatic maintenance costs km (eq. 5); the scaled reserve density e is given by eq. 6. Growth conditions are defined if the scaled reserve density exceeds the scaled length (l) (eq. 7). At conditions of low resource supply, the scaled reserve density may drop below the non-growth boundary and the animal is considered to be starving. There are several DEB starvation options available. For an overview see e.g. (Jager and Zimmer, 2012). However, it is reasonable to assume that upon starvation, reserve is used to pay somatic maintenance costs only. If assuming that the animal does not shrink in structure (weight loss due to reserve utilization is still possible) the mobilization flux is given by eq. 8, where κ is the reserve fraction that is allocated to soma.

The net result of reserve allocation to soma κ minus the somatic maintenance costs is used for growth, whereas the remaining fraction 1-κ is used for maturation and reproduction. The increase in structural length L is given by eq. 9, or, alternatively, if the amount of reserve does not allow the organism to growth, i.e. e ≤ l, then dL/dt = 0. For some species (abj and hep models), we apply the concept of metabolic acceleration to account for the initial slow growth observed during their ontogeny. For examples and a discussion of concepts see (Kooijman, 2014). Here, we assume that individuals accelerate their assimilation rate at some stage during their development, through changes in shape. A change in shape will most importantly alter the animal’s surface-area to volume ratio and thus its surface-area-specific assimilation rate, here pXm, as well the energy conductance v, which determines the reserve mobilization and is defined as the ratio of surface-area-specific assimilation rate and the maximum reserve density. For a brief derivation see (Zimmer et al., 2014). For modelling increased growth later during ontogeny, we apply an acceleration factor sM to the assimilation rate, the energy conductance and the specific feeding rate. sM is given by the ratio of the structural length of the organism divided by its structural length at birth. Once the scaled maturity at metamorphosis is reached, sM does not further increase and remains constant (structural length at metamorphosis divided by structural length at birth)

The change in scaled maturity UH depends on the 1-κ fraction of the mobilization flux Sc minus the maturity maintenance costs (eq. 10), where kj is the maturity maintenance rate. Fixed maturity levels mark the onset of feeding, the metamorphosis (i.e. end of acceleration), and the investment in reproduction. After reaching puberty (UPH), maturation stops and the mobilized reserve is allocated to the reproduction buffer (eq. 11) instead. The reproduction buffer is converted into a reproduction rate R with a fixed reproduction efficiency kr, depending on the energetic costs for producing a single egg (eq. 12).

Aging and starvation are considered as two major causes of natural death (additional mortality causes are described in section 7.12). Aging acceleration summarizes the production of damage producing compounds that are accumulated and copied at rates proportional to the rate of energy mobilization. Aging comes with two extra parameters, the aging stress coefficient sG and the aging acceleration constant ha (eq. 13); the resulting change in cumulative hazard H due to ageing is given by eq. 14.

While for the aging module we assume that damage accumulation is lower at lower food levels, at some point the food availability does not allow the organism to cover its maintenance costs and death by starvation occurs (Gergs and Jager, 2014)  . The survival probability s resulting from aging is given by eq. 15.

To address the influence of different temperatures on the life history processes, we multiplied the Arrhenius temperature correction factor to all metabolic and physiological rates (pX, pAm, dUH, dUE, dUR dL, dh, dq, s).

Table 7.3: Model formulations for representing energy acquisition and use in individual organisms.

| Equations | No. |
| --- | --- |
| Feeding and assimilation processes |  |
| pX = pXm * L2* x*(K+x)-1 | eq. 1 |
| pA = pX * kappa_x | eq. 2 |
| f = pA * (pAm * L²) -1 | eq. 3 |
| Reserve dynamics |  |
| dUEdt= f*L2*(pAm/pAm0)-SC | eq. 4 |
| SC=L2*g*e*(1+kM*L* υ-1 ) (g+e)-1 | eq. 5 |
| e = υ*UE* L-3 | eq. 6 |
| l = L*(vkm *g )-1 | eq. 7 |
| Sc=κ*kM*g*L3* υ-1 | eq. 8 |
| Growth |  |
| dL/dt=13L-2* (υ* g-1*SC – kM*L3) | eq. 9 |
| Maturation and reproduction |  |
| dUH/dt=1-κ*Sc - kJ UH for UH <UHp else dUH/dt=0 | eq. 10 |
| dUR/dt=1-κSc – kJ*UHp for UH ≥UHp else dUR/dt=0 | eq. 11 |
| R=kr*UR*UE0-1 | eq. 12 |
| Survival due to aging and starvation |  |
| dq/dt= q*L3* υ g-1 kM-1-3 s G+ ha*e*υ* L-1- 3 dL* L* dt-1- 3 dL* L*dt-1* q | eq. 13 |
| dh/dt=q - 3*dL L*dt-1*H | eq. 14 |
| s= exp(-h(t)) eq. 15 |  |

## 7.2	Process based TK-TD models for lethal and sublethal effects

Toxicokinetic-toxicodynamic (TK-TD) models allow for the meaningful analysis and interpretation of ecotoxicological data by quantifying toxicity and taking the processes leading to an effect into account. Therefore, TK-TD models facilitate the study of differential sensitivities of different species, and their stages, to different toxicants (Ashauer and Escher, 2010).

Toxicokinetics include not only the processes of uptake and elimination of chemicals but also their internal distribution and biotransformation, while toxicodynamics link the internal concentration to an effect at the organism level in a quantitative way (Jager et al., 2011).

Various existing TK-TD modelling approaches have been unified within the General Unified Threshold Model of Survival (GUTS) framework (Jager et al., 2011). For example, GUTS has been applied to predict survival under time-variable exposure (Nyman et al., 2013) (Ashauer et al., 2013), model starvation resistance (Gergs and Jager, 2014) and combined effects of toxicity and starvation (Nyman et al., 2012), represent temporal variation in toxicity in individual-based models (Gergs et al., 2013) (Gabsi et al., 2014), link temporal biomarker response to survival (Jager and Hansen, 2013), model survival of gill cells in vitro (Stadnicka-Michalak et al., 2015), approximate toxicodynamic recovery times (Ashauer et al., 2013) and investigate sensitivity differences between species and life-stages (Beaudouin et al., 2012) (Kulkarni et al., 2013) (Gergs et al., 2015).

The dose metric for an effect is typically the concentration of a toxicant in the medium surrounding an organism (e.g. water), the concentration inside the organism or a damage caused by the toxicant. In general, the choice of the most appropriate dose metric will depend on the available data and the hypotheses regarding the mechanism of action of the chemical of interest. This choice will affect the interpretation of the toxicodynamic parameters and their dimensions, for details see (Jager et al., 2011). In the absence of body residue data, the use of the scaled internal concentration as the dose metric is recommended (Jager et al., 2011). The scaled internal concentration is directly proportional to the actual (but unknown) internal concentration, and has the dimensions of an external concentration. The application of the (fitted) scaled internal concentration has been shown to provide meaningful results and even better fits to the survival data compared to the full GUTS explicitly including uptake and elimination as well as damage dynamics (Jager et al., 2011). However, toxicokinetic and toxicodynamic processes are combined and therefore the dominant rate constant will reflect the slowest compensating process (elimination or damage repair) dominating the overall dynamics of toxicity.

The (toxicokinetic) model for the dose metric can be linked with a survival model or models representing sublethal effects. For lethal effects, two special toxicodynamic cases within the GUTS framework, i.e. stochastic death (SD) and individual tolerance (IT), link the dose metric and survival. Within SD models, an organism has an increased probability to die when damage exceeds a threshold for survival. The hazard rate for an individual thereby increases linearly with the dose metric above its threshold for an effect. In contrast, in IT models, the threshold follows a frequency distribution within a population and death is instantaneous for an organism when damage exceeds the individual survival threshold. Consequently, in the IT models, half of a test population dies when the damage reaches the distribution median. The intrinsic properties of the two toxicodynamic assumptions result in different survival patterns over time: at constant exposure SD animals continuously die if the dose metric is beyond the threshold. While in SD models mortality increases with time at steady state, in the IT framework individuals within a population stop dying when the dose metric is in equilibrium. The two toxicodynamic assumptions of SD and IT can be seen as two extreme views on survival, most probably with the ‘real’ system behaviour in between.

DEB models offer a plausible and coherent way for the interpretation of chemical effects on life history processes such as growth and reproduction. Like other TK-TD approaches they integrate both exposure concentrations and time to yield a single set of parameters that can be used for further analyses such as the prediction of unknown exposure scenarios and, if integrated in an IBM framework, for the extrapolation of population level effects. First proposed by (Kooijman and Metz, 1984), the approach of interpreting toxicity data based on dynamic energy budgets resulted in the formulation of a simple ready to use model (Kooijman and Bedaux, 1996) which was referred to as DEBtox. Within DEBtox models several ways of how a chemical acts on life history processes are considered as a results of the interference with processes along the cascade of energy assimilation and allocation (Álvarez et al., 2006). An overview on these physiological modes of action is provided below. DEBtox and related models have been used to analyse toxicity data such as derived from growth and reproduction tests under constant exposure conditions (Jager et al., 2006) (Jager and Selck, 2011) (Goussen et al., 2015), or time-varying exposure (Pieters et al., 2006) and effects resulting from chemical mixtures (Jager et al., 2010).

A fully revised version of a simple DEB model to be used in ecotoxicology was presented by (Jager and Zimmer, 2012). The ‘simple’ refers to a reduced number of input parameters, compared to the formulation of a standard DEB model (Kooijman, 2010). This simplification, however, comes with a number of basic assumptions and limitations: 1) reserve is in a steady state with the food level, which is only realistic at conditions of constant food, 2) reproduction is initiated at a constant size irrespective of food availability and chemical exposure and 3) the energetic costs for producing one egg is constant, thus, maternal effects are not considered. More flexibility with regard to variable environmental conditions is possible by the use of the somewhat more complex standard DEB model (Kooijman, 2010) which explicitly accounts for maturation and uses maturation thresholds e.g. for puberty, thus, modelled individuals are allowed to reproduce at smaller size when food is not at ad libitum (for an example see (Gergs et al., 2014)). However, the concept of physiological modes of action is also applicable to the standard DEB model and the applicability of these models in an IBM framework have been demonstrated for chemical stress (Martin et al., 2013) and stress due to food limitation and crowding (Gergs et al., 2014)

For an overview on the equation for lethal and sublethal effects see Table 8.1. The change in scaled internal damage D* is given by eq. 20, where c is the external chemical concentration and kd the elimination rate constant (also called the dominant rate constant). In addition to the state of the art GUTS framework, a temperature correction (fT) based on the DEB temperature parameters is considered for the calculation of the internal damage. In accordance with (Jager, 2013) and (Gergs et al., 2015), body size-dependent toxicokinetics are introduced as an extension of the GUTS framework (eq. 21). The elimination rate kd is scaled with the surface to volume ratio 1/L, relative to the maximum ratio, and therefore, the factor Lm/L is introduced, where L denotes the actual structural length, Lm the maximum structural length an animal can reach and kd* the scaled elimination rate. Introducing Lm removes the dimension of length or weight from the equation. The absolute value for kd in eqn. 21 can be interpreted as the elimination rate when an organism reaches its maximum length. This size scaling extension to the GUTS framework has previously been tested for different aquatic invertebrate species such as Daphnia magna, Chaoborus crystallinus, and Mesocyclops leuckartii (Gergs et al., 2015).

For survival modelling, the toxicodynamic assumption of SD is employed which is one of the two special cases in the GUTS framework. Therefore, the change in cumulative hazard h is calculated based on the scaled internal damage (eq. 22), the threshold concentration zs and the effect strength bs (eq. 22). The corresponding survival probability S is given in eq. 23.

Table 8.1: Model formulations for representing lethal and sublethal effects.

| Equations No. | No. |
| --- | --- |
| D*dt= kd c-D*fT | eq. 20 |
| kd*=kd*Lm* L-1 | eq. 21 |
| dh/dt=bs max ( D*-zs, 0) | eq. 22 |
| S(t)= exp (-H(t)) | eq. 23 |
| s=bb* max (D* -zb, 0) | eq. 24 |

For modelling sublethal effects on individual growth and reproduction, the DEB concept of physiological modes of action is employed (Álvarez et al., 2006) which includes several options for an effect. In this model, it is assumed that upon toxicant exposure the organisms stress level s increases beyond an effect threshold zb in proportion to the toxicant concentration. Therefore, a stress function is applied to the different DEB parameters (eq. 24), thus parameter values change proportional to the toxicant concentration, with the slope of the response being determined by the effect strength bb. The parameters that change upon chemical exposures depend on the physiological mode of action considered for an effect (Table 8.2). We here illustrate parameter changes for an effect on feeding (eq. 25), on maintenance costs (eq. 26), on costs for growth (eq. 27) and a hazard during oogenesis (eq. 28).

Table 8.2: Physiological modes of action used for sublethal effect modelling

| Equations | No. |
| --- | --- |
| f → f*max(0, 1-s) | eq. 25 |
| km → km* 1+s ; kj → kj*(1+s) | eq. 26 |
| km → km* (1+s) ; g → g*(1+s) | eq. 27 |
| kr → kr* exp(-s) | eq. 28 |

Models for survival and feeding inhibition will be based on the same toxicokinetics, implying that we, in terms of the mechanism, assume that the same internal concentration or damage will cause both lethal and sublethal effects. Effects might occur at different levels of concentrations though, i.e. model wise the threshold for the scaled internal concentration for survival (zs) and sublethal effect (zb) might differ. In turn, assuming that damages for lethal and sublethal effects differ would require the calculation of two different scaled internal concentrations with different values for the elimination rate constants. Besides time resolved data on survival, we would need observations on feeding rate over time for model parametrization, however, most of the data sets come with only one measurement over time.

The same applies to the application of the IT toxicodynamic assumption within the GUTS framework. The GUTS-IT works with a median for the distribution of the threshold within a population, similar to an EC50 value, while GUTS-SD and the stress function for sublethal effects are based on thresholds for an individual organism, which are conceptually identical to a no-effect-concentration. As combining a DEB model for feeding inhibition with GUTS-IT would also require different toxicokinetic models, in this project, we only apply the GUTS-SD toxicodynamic assumption for modelling lethal effects.

## 7.3	Calculation of initial resources

For the simulated herbivore species, the model provides four types of food resources, the periphyton compartment and three different fractions of particulate organic matter. On the first simulation day the initial periphyton energy density is calculated for each aquatic cell of the water section (eq. 7.3.1).

periphytond=periphytonK*periphyton E			(7.3.1)

periphytond = periphyton density in cell [J/m2]

periphytonK = periphyton capacity [g/m2]

periphytonE = Periphyton energy density [J/g]

The default value for the periphyton energy density is set to 4623 J/g (McIntire and Phinney, 1965), and the default value for the periphyton capacity is 0.4 g/m2.

The initial C-POM content for each aquatic cell with a slow velocity or less is set to the selected value. These slow velocity cells also define the areas in the water body where leaf litter can accumulate. In the future, it is planned to integrate a method that is able to calculate the initial C-POM density depending on the start date of the simulation, the canopy cover of the site and thus the resulting decomposition.

For further calculations, the amount of C-POM per cell is converted into the energy density per cell (eq. 7.3.2)

CPomd=CPomI*CPomE					(7.3.2)

CPomd = CPom density in cell [J/m2]

CPomI = initial CPom density [g/m2]

CPomE = CPom energy density [J/g]

The default value for the C-POM energy density is set to 7227.5 J/g (Lamprecht, 1999).

The initial energy densities for F-POM and S-POM are not calculated, instead these parameters are fixed values, set in the environmental settings window. The default value for F-POM density is 80000 J/m2, the value for the S-POM density is 500 J/m2. F-POM and S-POM, just like the C-POM, are only initialized in aquatic cells with a velocity category of slow or less.

All the described environmental parameters can be changed by the user before starting the simulation.

## 7.4	Update resource densities

Once per day the resource densities are updated.

A generic periphyton compartment is considered as food source for grazing macroinvertebrates. We use a logistic growth model to simulate seasonal periphyton dynamics depending on a temperature function (based on the ambient water temperature) and a light function (considering the amount of light by using the annual cycle of the photoperiod), which both show typical patterns during the year. The modelled periphyton growth also depends on shading from the overlay of coarse particular organic matter (C-POM, see below) such as leaf litter, resulting in a spatially heterogeneous periphyton distribution.

We consider the input of leaf litter as the only contribution to the C-POM compartment, which serves as food source for shredders. Leaf litter entry in autumn depends on the stream area covered by trees as well as on the (decreasing) daylight length. Shredders only partially consume their food. Leftovers from shredding activities as well as faeces and animal corpses contribute to fine particular organic matter (F-POM), assuming immediate decomposition for simplicity. F-POM serves as resource for deposit feeders. Both, F-POM and C-POM, can accumulate in slow-flowing sections of the water body.

The third fraction of POM, suspended particulate organic matter (S-POM) is (partially) available for benthic filter feeders.

## 7.5	Calculation of periphyton growth

For the herbivore species simulated in the model, there are two different pools of food resources: periphyton and particulate organic matter. The periphyton compartment serves as food source for species with grazing feeding preference. The periphyton growth rate depends on the Arrhenius temperature and on the Leaf Area Index. In a first step, the metabolic rate of the periphyton is calculated based on the Arrhenius temperature (eq. 7.5.1).

Currently, we have not implemented any influence of the nutrient content on the growth rate of the periphyton. Since nutrient availability is a main driver for the growth of the periphyton, the implementation of this process should be done in the next development steps.

fT=e(TA20+273.15-(TAtempd+273.15))				(7.5.1)

fT = metabolic rate at temp [-]

TA = periphyton Arrhenius temperature [K]

tempd = temperature of the day [K]

The used Leaf area index (LAI) is based on the work of (Tillack et al., 2014)(7.5.2) for an alluvial forest.

LAI≔4.8638-4.8638*e-0.5d-213.725260.3975,2				(7.5.2)

d = simulation day of year

Since the individual map cells are shaded differently by the canopy, the LAI for each cell is corrected by a shading factor and the photoperiod of the simulated day (Forsythe et al., 1995) (eq. 7.5.3, 7.5.4), resulting in the light factor fL.(eq. 7.5.5)

p=arcsin0.39795*cos0.2163108 + 2*arctan0.9671396*tan0.0086*d-186		(7.5.3)

p = daylength coefficient[-]

d = day of the year

photoperiodh=24-24pi*arccos⁡(sin0.8333*pi180+sinL*pi180*sin⁡(p)cosL*pi180*cos⁡(P))		(7.5.4)

L = latitude

p = day length coefficient

fL[1/d]≔ LAI*shading factor*photoperiodh24			(7.5.5)

fL = light rate [-]

LAI = leaf area index [-]

To calculate the periphyton growth the following logistic growth model is used (eq. 7.5.6).

periphytond= periphytond+ r *fT*fL* periphytond* 1-periphytondk(7.5.6)

periphytond = periphyton density in cell

r = periphyton growth rate [d-1]

fT = temperature correction [-]

fL = light correction [-]

k = periphyton capacity [g/m²]

The minimum periphyton density per cell is set to 0.000001 g/m2, prohibiting the periphyton compartment to go extinct in a given cell. On the other hand, it must not exceed the fixed maximum periphyton carrying capacity. If the density of the coarse particulate matter is more than 10000 g /m2, there is no periphyton growth at all, because the particulate matter insulates the periphyton from light.

## 7.6	Calculation of particulate organic matter

The second food source for herbivores besides the periphyton is particulate organic matter (POM). The model simulates three different categories of POM.

The only input for coarse particulate organic matter (CPOM) is the leaf litter entry (eq. 7.6.1). Currently, the annual litter fall is initialized at simulation day 270 which is the 16th of October, while the simulation starts at January 1st.

In further model development, we will initialize the beginning of the litter fall depending on the season. When the photoperiod is less than 10.7 and the length of the simulated days gets shorter, the entry of CPOM will start. Another aspect we should consider, is the canopy cover of the simulated site, which has also an important impact on the leaf litter amount.

The litter input is only initialized for aquatic cells with a velocity category of slow or less. Formula 7.6.2 shows the calculation of litter input in detail.

CPomd=CPomd+ litterE *CPomS			(7.6.1)

CPomd = CPom density in cell [g/m²]

litterE = leaf litter entry [g/m²]

CPomS = CPom settlement rate [1/d]

The following calculation of the leaf litter entry depends on a regression from (Benfield, 1997)

litterE=790.9-10.8*latitude				(7.6.2)

litterE = initial leaf litter density [g/m²]

After calculating the C-POM per m2, it is necessary to convert this value to the cell size (7.6.3).

CPomd=(CPomd*cellsize+ FPomS *DeadBodies/NumberSlowCells)/cellsize (7.6.3)

CPomd = CPom density in cell [g/m²]

FPomS = FPom settlement rate [1/d]

The fine particulate organic matter accumulates only in cells with slow velocity or less. The only input for F-POM is suspended particulate matter (S-POM) (7.6.4).

FPomd≔ FPomd+FPomS*SPomd			(7.6.4)

FPomd = FPom density in cell [g/m²]

FPomS = FPom settlement rate [1/d]

SPomd = SPom density in cell [g/m²]

Residues of feeding activity are considered as suspended particulate matter. The density of S-POM is thus calculated from the sum of faeces divided by the total area of aquatic cells as shown in formula 7.6.5.

SPomd≔SummedFaeces*cellsizeAreaAquaticCells		(7.6.5)

SPomd = SPom density in cell [g/m2]

The consummation of resources is calculated as part of the procedure for calculating the feeding rates per species population.

At its current state, the feeding procedure is the only process which reduces POM density. Other reducing processes of this compartment such as microbial decomposition are not implemented, yet.

## 7.7	Movement

The individual attempts moving to another cell if a random number exceeds the probability to stay. The probability to stay is calculated from the substrate preferences of the individual, the depth, the velocity of the cell and the energy content within the individual (7.7.1).

Pstay=maxMP*OP*VP*DP*e				(7.7.1)

Pstay  = probability to stay in cell [-]

MP = preference for mineral substrate [-]

OP = preference for organic substrate [-]

VP = preference for velocity class [-]

DP = preference for depth class [-]

e = scaled reserve density [-]

If the individual decides to move, it randomly selects an adjacent cell within the waterbody.

## 7.8	Feeding herbivores

The individual feeding process is influenced by two main drivers: the selected food source and the corresponding food density.

7.8.1	Foraging

The selected food type is determined by the food energy in the cell and the feeding preferences of the individual (figure 7.2). If the resulting food type is the periphyton compartment, only the edible fraction is reduced by the calculated feeding rate. If the resulting food type is the S-POM, only the available fraction for filterer is considered as food source.

Figure 7.2: Decision tree food source

7.8.2	Feeding

As long as the food density on a cell is greater zero, each individual staying on that cell undergoes the feeding process as described below.

The hourly feeding rate is calculated in random order based on the feeding rate and the half saturation constant (eq. 7.8.1).

px := (pXm24* L2) *(fooddensity / (K + fooddensity)		(7.8.1)

px = feeding rate per hour [/h]

pXm = maximum specific feeding rate [J/(d * cm2)]

L = length of individual [cm]

K = half saturation constant  [J]

At the end of the day, the hourly feeding rate is added up to daily feeding rate, taking into account a temperature correction (eq. 7.8.2).

pXsum := pXsum + pX * f_T				(7.8.2)

pXsum = daily feeding rate [/d]

px = feeding rate per hour [/h]

f_T = temperature correction [-]

Based on that daily feeding rate, the scaled functional response (ingestion rate) is calculated (eq. 7.8.3).

f := pXsum * kappa_x / (pAm * f_T * L²)		(7.8.3)

f = scaled functional response [-]

pXsum = daily feeding rate [/d]

kappax = digestion efficiency food to reserve [-]

pAm = assimilation rate [J/(d * cm2)]

f_T = temperature correction [-]

L = length of individual [cm]

Further information related to the used feeding formulas can be found in section 7.1 (DEB theory)

The resource density in the cell is reduced by the summed feeding rate of all organisms that reside on that specific cell. Faeces and leftovers of the feeding activity are added to the total faeces amount (7.8.4).

feaces=faeces+px*kappaX				(7.8.4)

px = feeding rate [/h]

kappax = digestion efficiency food to reserve [-]

## 7.9	Predation

For a cell where a hatched predatory individual is located, the body energy density of all prey individuals smaller than 10 % of the predator’s body size is added to the food density in that specific cell (7.9.1). When the food density in the cell is positive, the predator runs through the feeding process. After feeding, faeces of the predator and the remains of the feeding activity are added to the summed faeces. As long as there are still prey individuals in the cell and the feeding rate of the predator is greater than the already ingested energy, the prey individuals are eaten and die. After the feeding process the predator continues with its move procedure. The schedule of the predation process is shown in figure 3.1.

fooddensity≔ fooddensity+ xiWE*1000 * L3*w+Ur*pAM*siblings	(7.9.1)

XiWE = whole-body energy density (without reproduction buffer) [kJ/g]

L3 = body volume of prey [mm3]

w = density of dry weight [g/cm3]

Ur = scaled reproduction buffer [d*cm2]

pAm = maximum specific assimilation rate [J/(d*cm2)]

Figure 7.3: Flow chart predation

## 7.11	Reproduction

Once an individual reached maturity, the reproduction process is initialised. For a few species, the timing of reproduction also depends on the photoperiod, and they only start to reproduce if the photoperiod of the day is sufficient. Depending on the species, we have implemented three different reproduction strategies:

seasonal iteroparous reproduction, semelparous reproduction and viviparous reproduction.

The viviparous reproduction strategy applies to species that reproduce continuously by forming new eggs inside their bodies as soon as the previous brood has been released. The size of a new brood depends on the size of the reproduction buffer. Formula 7.11.1 shows the detailed calculation.

R=kr*URU0E			   			(7.11.1)

R = brood size [#/d]

kr = reproduction efficiency [-]

UR = scaled reproduction buffer [d*cm²]

U0E = cost of an individual egg/embryo [J]

The development of an embryo resembles that of its parent animal with the important difference that it does not consume food. The scaled catabolic flux (eq. 7.11.2) and the growth rate (eq. 7.11.3) of the embryo are calculated under the assumption that parameter values are inherited. The scaled reserve density of the embryo decreases over time while it grows and matures (eq. 7.11.4). If the embryo reaches the scaled maturity threshold for birth, the breeding procedure stops and the hatched offspring is released and starts feeding.

bSc=bL2*(g0*beg0+be*(1+km0*bLv)			(7.11.2)

bSc = scaled catabolic flux of the embryo [J / d*cm²]

bL = structural length of the embryo [cm]

g0 = energy investment ratio [-]

be = scaled reserve density of the embryo [-]

km0 = somatic maintenance rate coefficient [per d]

v = energy conductance [mm/d]

bL=bL+13*bL2*(vg0*bSc-km0*b_L³)step				(7.11.3)

bL = structural length of the embryo [cm]

v = energy conductance [mm/d]

g0 = energy investment ratio [-]

bSc = scaled catabolic flux of the embryo [J / d*cm²]

km0 = somatic maintenance rate coefficient [per d]

step = time steps [per day]

bUH=bUH+((1-kappa0)*b_Sc )-(kj-bUH)step			(7.11.4)

bUH = scaled maturity of an animal in brood [d * cm²]

kappa0 = allocation fraction to soma [-]

bSc = scaled catabolic flux of the embryo [J / d*cm²]

kj = maturity maintenance rate coefficient [per d]

step = time steps [per day]

Semelparous reproduction is the reproduction strategy of insects with terrestrial adults. Depending on the dispersal rate, there is only a specific percentage of terrestrial adults that reproduce within the system. It is assumed that the other individuals leave the simulation area and lay their eggs outside the system. Regardless of the reproduction location the adult individual dies after completing the reproduction process. During this process the brood size is calculated (eq. 7.11.1), and the resulting number of offspring is released. It should be noted that with this reproduction strategy, individuals reproduce only once (typically with a considerable number of offspring), as opposed to continuous reproduction with multiple broods.

If the reproduction of the species depends on the seasons, the seasonal iteroparous reproduction is simulated. The brood size of these species is calculated in the same way as for the semelparous species (eq. 7.11.5). However, in contrast to the semelparous individuals the adults do not die after the reproduction process.

The eggs laid undergo the developmental processes described for the embryos of viviparous species (eq. 7.11.2 - 7.11.4). If offspring is released, the model produces new individuals on the same cell their mother is located and adds these to the community. The newly created individuals are not affected by the toxic load of their mothers, so they start with an internal concentration of zero. All newly laid eggs are initialized with a structural length of 0.000001.

## 7.12	Survival

Death can occur because of aging, starving, background mortality, or chemical exposure. The aging and starvation processes are part of the DEB calculations and are further described in section 7.1. The chemical induced hazard and its influence on mortality is calculated based on the TKTD-SD approach (see 7.2).

All mortality rates, with exception of the background mortality, are temperature dependent and increase with higher temperatures. The density dependent as cause of mortality and its temperature dependence can be separately switched on or off. Further information of the stochastic aspects related to survival probability, are described in section 4.6.

- References

Álvarez, O.A., Jager, T., Redondo, E.M., Kammenga, J.E., 2006. Physiological modes of action of toxic chemicals in the nematode Acrobeloides nanus. Environmental Toxicology and Chemistry 25, 3230. https://doi.org/10.1897/06-097R.1

Ashauer, R., Escher, B.I., 2010. Advantages of toxicokinetic and toxicodynamic modelling in aquatic ecotoxicology and risk assessment. J. Environ. Monit. 12, 2056. https://doi.org/10.1039/c0em00234h

Ashauer, R., Thorbek, P., Warinton, J.S., Wheeler, J.R., Maund, S., 2013. A method to predict and understand fish survival under dynamic chemical stress using standard ecotoxicity data. Environmental Toxicology and Chemistry 32, 954–965. https://doi.org/10.1002/etc.2144

Beaudouin, R., Zeman, F.A., Péry, A.R.R., 2012. Individual sensitivity distribution evaluation from survival data using a mechanistic model: Implications for ecotoxicological risk assessment. Chemosphere 89, 83–88. https://doi.org/10.1016/j.chemosphere.2012.04.021

Benfield, E.F., 1997. Comparison of Litterfall Input to Streams. Journal of the North American Benthological Society 16, 104–108. https://doi.org/10.2307/1468242

Forsythe, W.C., Rykiel Jr, E.J., Stahl, R.S., Wu, H., Schoolfield, R.M., 1995. A model comparison for daylength as a function of latitude and day of year. Ecological Modelling 80, 87–95.

Gabsi, F., Hammers-Wirtz, M., Grimm, V., Schäffer, A., Preuss, T.G., 2014. Coupling different mechanistic effect models for capturing individual- and population-level effects of chemicals: Lessons from a case where standard risk assessment failed. Ecological Modelling 280, 18–29. https://doi.org/10.1016/j.ecolmodel.2013.06.018

Gergs, A., Jager, T., 2014. Body size-mediated starvation resistance in an insect predator. Journal of Animal Ecology 83, 758–768. https://doi.org/10.1111/1365-2656.12195

Gergs, A., Kulkarni, D., Preuss, T.G., 2015. Body size-dependent toxicokinetics and toxicodynamics could explain intra- and interspecies variability in sensitivity. Environmental Pollution 206, 449–455. https://doi.org/10.1016/j.envpol.2015.07.045

Gergs, A., Preuss, T.G., Palmqvist, A., 2014. Double Trouble at High Density: Cross-Level Test of Resource-Related Adaptive Plasticity and Crowding-Related Fitness. PLoS ONE 9, e91503. https://doi.org/10.1371/journal.pone.0091503

Gergs, A., Zenker, A., Grimm, V., Preuss, T.G., 2013. Chemical and natural stressors combined: from cryptic effects to population extinction. Scientific Reports 3. https://doi.org/10.1038/srep02036

Goussen, B., Beaudouin, R., Dutilleul, M., Buisset-Goussen, A., Bonzom, J.-M., Péry, A.R.R., 2015. Energy-based modelling to assess effects of chemicals on Caenorhabditis elegans: A case study on uranium. Chemosphere 120, 507–514. https://doi.org/10.1016/j.chemosphere.2014.09.006

Grimm, V., Railsback, S.F., Vincenot, C.E., Berger, U., Gallagher, C., DeAngelis, D.L., Edmonds, B., Ge, J., Giske, J., Groeneveld, J., Johnston, A.S.A., Milles, A., Nabe-Nielsen, J., Polhill, J.G., Radchuk, V., Rohwäder, M.-S., Stillman, R.A., Thiele, J.C., Ayllón, D., 2020. The ODD Protocol for Describing Agent-Based and Other Simulation Models: A Second Update to Improve Clarity, Replication, and Structural Realism. Journal of Artificial Societies and Social Simulation 23. https://doi.org/10.18564/jasss.4259

Hendriks, A.J., 1999. Allometric Scaling of Rate, Age and Density Parameters in Ecological Models. Oikos 86, 293–310. https://doi.org/10.2307/3546447

Holling, C.S., 1959. The Components of Predation as Revealed by a Study of Small-Mammal Predation of the European Pine Sawfly. The Canadian Entomologist 91, 293–320. https://doi.org/10.4039/Ent91293-5

Jager, T., 2013. All Individuals Are Not Created Equal; Accounting for Interindividual Variation in Fitting Life-History Responses to Toxicants. Environ. Sci. Technol. 130111083350000. https://doi.org/10.1021/es303870g

Jager, T., Albert, C., Preuss, T.G., Ashauer, R., 2011. General Unified Threshold Model of Survival - a Toxicokinetic-Toxicodynamic Framework for Ecotoxicology. Environ. Sci. Technol. 45, 2529–2540. https://doi.org/10.1021/es103092a

Jager, T., Hansen, B.H., 2013. Linking survival and biomarker responses over time: Linking survival and biomarker responses over time. Environmental Toxicology and Chemistry 32, 1842–1845. https://doi.org/10.1002/etc.2258

Jager, T., Heugens, E.H.W., Kooijman, S.A.L.M., 2006. Making Sense of Ecotoxicological Test Results: Towards Application of Process-based Models. Ecotoxicology 15, 305–314. https://doi.org/10.1007/s10646-006-0060-x

Jager, T., Selck, H., 2011. Interpreting toxicity data in a DEB framework: A case study for nonylphenol in the marine polychaete Capitella teleta. Journal of Sea Research 66, 456–462. https://doi.org/10.1016/j.seares.2011.04.003

Jager, T., Vandenbrouck, T., Baas, J., De Coen, W.M., Kooijman, S.A.L.M., 2010. A biology-based approach for mixture toxicity of multiple endpoints over the life cycle. Ecotoxicology 19, 351–361. https://doi.org/10.1007/s10646-009-0417-z

Jager, T., Zimmer, E.I., 2012. Simplified Dynamic Energy Budget model for analysing ecotoxicity data. Ecological Modelling 225, 74–81. https://doi.org/10.1016/j.ecolmodel.2011.11.012

Jeschke, J.M., Kopp, M., Tollrian, R., 2002. Predator Functional Responses: Discriminating Between Handling and Digesting Prey. Ecological Monographs 72, 95–112. https://doi.org/10.1890/0012-9615(2002)072[0095:PFRDBH]2.0.CO;2

Kooijman, S.A.L.M., 2014. Metabolic acceleration in animal ontogeny: An evolutionary perspective. Journal of Sea Research, Dynamic Energy Budget theory: applications in marine sciences and fishery biology 94, 128–137. https://doi.org/10.1016/j.seares.2014.06.005

Kooijman, S.A.L.M., 2010. Dynamic energy budget theory for metabolic organisation, 3. ed. Cambridge Univ. Press, Cambridge.

Kooijman, S.A.L.M., Bedaux, J.J.M., 1996. Analysis of toxicity tests on Daphnia survival and reproduction. Water Research 30, 1711–1723. https://doi.org/10.1016/0043-1354(96)00054-1

Kooijman, S.A.L.M., Metz, J.A.J., 1984. On the dynamics of chemically stressed populations: The deduction of population consequences from effects on individuals. Ecotoxicology and Environmental Safety 8, 254–274. https://doi.org/10.1016/0147-6513(84)90029-0

Kulkarni, D., Daniels, B., Preuss, T.G., 2013. Life-stage-dependent sensitivity of the cyclopoid copepod Mesocyclops leuckarti to triphenyltin. Chemosphere 92, 1145–1153. https://doi.org/10.1016/j.chemosphere.2013.01.076

Lamprecht, I., 1999. Combustion calorimetry, in: Handbook of Thermal Analysis and Calorimetry. Elsevier, pp. 175–218.

Martin, B.T., Jager, T., Nisbet, R.M., Preuss, T.G., Grimm, V., 2013. Predicting Population Dynamics from the Properties of Individuals: A Cross-Level Test of Dynamic Energy Budget Theory. The American Naturalist 181, 506–519. https://doi.org/10.1086/669904

McIntire, C.D., Phinney, H.K., 1965. Laboratory studies of periphyton production and community metabolism in lotic environments. Ecological Monographs 35, 237–258.

Nyman, A.-M., Hintermeister, A., Schirmer, K., Ashauer, R., 2013. The Insecticide Imidacloprid Causes Mortality of the Freshwater Amphipod Gammarus pulex by Interfering with Feeding Behavior. PLoS ONE 8, e62472. https://doi.org/10.1371/journal.pone.0062472

Nyman, A.-M., Schirmer, K., Ashauer, R., 2012. Toxicokinetic-toxicodynamic modelling of survival of Gammarus pulex in multiple pulse exposures to propiconazole: model assumptions, calibration data requirements and predictive power. Ecotoxicology 21, 1828–1840. https://doi.org/10.1007/s10646-012-0917-0

Pieters, B.J., Jager, T., Kraak, M.H.S., Admiraal, W., 2006. Modeling responses of Daphnia magna to pesticide pulse exposure under varying food conditions: intrinsic versus apparent sensitivity. Ecotoxicology 15, 601–608. https://doi.org/10.1007/s10646-006-0100-6

Press, W.H., Press, W.H., Flannery, B.P., Teukolsky, S.A., Vetterling, W.T., Flannery, B.P., Vetterling, W.T., 1989. Numerical Recipes in Pascal (First Edition): The Art of Scientific Computing. Cambridge University Press.

Stadnicka-Michalak, J., Schirmer, K., Ashauer, R., 2015. Toxicology across scales: Cell population growth in vitro predicts reduced fish growth. Sci. Adv. 1, e1500302. https://doi.org/10.1126/sciadv.1500302

Tillack, A., Clasen, A., Kleinschmit, B., Förster, M., 2014. Estimation of the seasonal leaf area index in an alluvial forest using high-resolution satellite-based vegetation indices. Remote Sensing of Environment 141, 52–63.

van der Meer, J., 2006. An introduction to Dynamic Energy Budget (DEB) models with special emphasis on parameter estimation. Journal of Sea Research 56, 85–102. https://doi.org/10.1016/j.seares.2006.03.001

Zimmer, E.I., Ducrot, V., Jager, T., Koene, J., Lagadic, L., Kooijman, S.A.L.M., 2014. Metabolic acceleration in the pond snail Lymnaea stagnalis? Journal of Sea Research 94, 84–91. https://doi.org/10.1016/j.seares.2014.07.006